Finding a dump station can be a daunting task sometimes so it is best to plan ahead.  (no pun intended)

The “Dump Stations” App

Lake Sammamish State Park

Sanidumps: Finding RV Dump Stations near Zip code 98027

Sanidumps.com Dump Station Map for Washington

NOTE: It is best to empty the cassette toilet more frequently than to wait until it is full.  When you locate a suitable location or when dumping the gray tank take the opportunity to empty the toilet also.  I have found it is way less stressful the maintenance task when it is convenient vs when you have to.

# Empty Casset Toilet:

- Preparation: Gather your tools—gloves, a bucket, and a sense of adventure. 🌟

- Water Off, Friends Not: Before you dive into cassette toilet wizardry, don’t forget to kindly inform your camping companions: “Toilet under construction—please consider other options!” 🚰🌲

- Access the Cassette: Locate and open the exterior door of your RV and pull out the cassette holding tank. It’s like pulling out a secret treasure chest! 🗝️

- Release the Waste: Unclip the waste tank from the fresh flushing tank (slightly different on each model, but you’ll spot it). Unscrew the lid from the drainage pipe on the toilet and empty the waste into the drain at the service point if at a dump station.  If using a toilet pipe very close to bowl and dump in toilet in one motion.  Only flush the toilet when tank is empty. 🚽

- Rinse and Shake: Rinse the cassette by filling it with water, replacing the lid, and giving it a joyful shake! 💃 Then again pour tank into toilet or drain.

- Add water: Add about 2 cups of water and one scoop of Happy Camper to help keep those odors away.

- Slide It Back In: Slide the cleaned cassette back into its cozy home. Secure it with a triumphant click. 🎉

- Water On, Adventure Continues: Finally, turn the water supply back on, and let the flushing fun begin anew! 💦🚐

# Emptying the GRAY TANK:

Let’s dive into the delightful world of gray water tanks. These tanks hold the water from your RV’s shower and kitchen sink—basically, all the non-toilet liquid magic. Here’s how to gracefully drain the gray water tank:

- Position Your RV: Pull or back up to the dump station’s disposal tank. Make sure your sewer hose reaches comfortably with a little slack to spare.  Ideally you are about 4-5 feet away.

- Find the Gray: I prefer to first securely Remove the cap from the outlet port that connects the black and gray water tanks. Connect your sewer hose to the gray water tank’s relief valve. Lead the hose to the input of the dump station’s disposal tank.

- Let It Flow: Open the relief valve and let the gray water pour out. To keep the hose steady, you can place a rock or brick on it (much better than tap dancing on it yourself!). 🌊

Remember, gloves are your trusty sidekicks during this process! If you need more RV wisdom, just give me a holler. Happy tank-taming! 🚐💧🌟 1

# Filling the Propane tank

Propane really goes a long way unless it is the dead of a “Seattle” winter and the heater is running a lot.  In summer where you are just using hot water and cooking a tank will last a couple weeks.

- Check The Tank: First, determine how much your propane tank can hold.  All propane tanks can only be filled to 80% of the tank’s capacity.  If there is a dial it will be about 3/4 when full.  Empty can be determined when there is no longer any propane in the tank also known as losing your fire. The tank holds about 5 actual gallons total so ¼ would be 1.25 gallons.

- Locate a station:  This can be challenging at times so be patient if you are out of the area.  For Issaquah area I prefer to fill up at The Grange.  On the Renton side there are a couple good options.   Mobile   Shell

- Park and Prepare: Pull up to the big propane tank at the station.  The valve is under the main sliding door so try and park within 10 feet of that location and the propane pump.  Turn off Luna and turn off the propane using one of the two switches.  Have other companions exit the RV.  Most of the time there will be a button to push for help or you will need to go into the station and ask if you could get some propane for your RV. The attendant will take it from there.

- Easy Peasy Tank Victory: Your tank is now full— 🚐💨

Hi copilot! Hope your having a great day! I need to make a word document. The topic will be how to change a cassette toilet. You will number each step with key photos of certain parts. The tone of the doc should be upbeat and fun.