2019 Sprinter Chassis — Charger Cycling & Wipers Activating: Troubleshooting Notes

Symptom observed: connecting a 6A battery charger causes the windshield wipers to activate on their own, and the charger goes to full load for ~30 seconds, then cuts off, then comes back on — repeating.

Update: the installed battery is an H6, not the H8 this Sprinter needs

On inspection, the currently installed battery is a Group 48/H6, while this Sprinter is specced for Group 49/H8 (see Winnebago_View_2020_Chassis_Battery_Replacement.md → H8 vs. H6). This is now the leading suspect for the symptoms above, ahead of the SAM/ground-strap theory below — a genuinely simpler and more likely explanation.

H8 is about 2.9" longer than H6 (same width/height). Fitting an H6 into an H8 tray needs a mounting bracket/spacer to take up that extra length and let the factory hold-down clamp actually grip the case.

Confirmed: a ~3" bracket is present and holding the battery in place. That rules out the simplest version of this theory (battery just loose in an oversized tray). Two things still worth a quick check before moving on:

Is the bracket actually snug, or does the battery have any wiggle when you push on it? A bracket that's present but not fully tightened can still allow enough movement to cause an intermittent connection under load.

Do the terminal cables reach without strain, or are they extended/spliced to bridge the gap? A marginal splice or a taut cable can be a connection point that only fails under the higher current a charger pushes, even with the case itself mechanically secure.

If both of those check out fine, the mounting angle is likely resolved, and the isolation test in Step 3 below becomes the key next move — it separates "the battery itself is genuinely bad" (possible after however long it's been running mismatched against this Sprinter's H8-spec charging system) from "it's a SAM/ground-strap issue" as covered further down.

What the SAM module is

SAM = Signal Acquisition Module (sometimes called Signal Acquisition and Actuation Module). Mercedes has used them since the mid-90s, and most Mercedes vehicles have 4–5 of them. Think of it as a network router for the vehicle's electrical system: it receives input from switches, sensors, and controllers, and sends output to actuate components (lights, wipers, locks, etc.) — replacing what would otherwise be hundreds of discrete point-to-point wires with a shared data network. When you press a switch, the signal passes through a SAM before the component responds. A fault in a SAM (rather than in the component itself) is why symptoms often look scattered and unrelated — wipers, lights, locks, warning messages — because they all route through the same module.

Location on the Sprinter: the front SAM sits near the base of the passenger-side A-pillar, low in that footwell/kick-panel area — this is also the spot SprinterRVDesk specifically flags as a moisture-intrusion point (water entering via the A-pillar or windshield seal corrodes the SAM's connectors over time). Since this Sprinter is under an RV conversion, trim panels may cover direct access — treat this as the area to start with, not necessarily a panel you can pop off in two minutes; confirm exact access against the Sprinter service manual (WIS) or have a tech locate it if it's not obviously reachable.

Most likely explanation

Applying a charger pushes more current through the system than normal resting draw. A SAM connector or ground strap that "mostly works" under light load can glitch or trip under that extra current — which would explain both the wipers activating (a SAM/CAN-bus glitch triggered by the current) and the charger cycling off every ~30 seconds (its protection circuit reacting to the fault it's seeing through that same bad connection).

Ground strap corrosion is the other half of this: every electrical circuit in the van returns current through the ground straps (engine-to-frame, battery negative-to-chassis, chassis-to-body). A corroded strap adds resistance to that shared return path, which can produce simultaneous, seemingly unrelated symptoms — hard starting, alternator warnings, random check-engine lights, instrument cluster malfunctions — alongside exactly the kind of wiper/charger behavior described here.

Diagnostic sequence (free, ~90 minutes, before buying any parts)

1. Inspect all ground connections — engine-to-frame, battery negative-to-chassis, chassis-to-body. Clean to bare metal with a wire brush, apply dielectric grease. Replace the strap ($15–25) if corroded or frayed.

2. Check the SAM module's connectors for corrosion (the A-pillar moisture-entry point above) — clean with electrical contact cleaner and a brass brush, reapply dielectric grease.

3. Check the fuse box's rear connectors for corrosion — another known weak point.

4. Audit any aftermarket/RV-conversion wiring for taps or splices near these areas.

5. Inspect the sliding door harness boots.

6. Read fault codes with a Mercedes-compatible scan tool if the above doesn't resolve it — see Sprinter_OBD_Scanner_Review.md.

Check the charging connection point

VS30 Sprinter owners note the correct chassis-battery charging point is the under-hood jump-start posts, not the AGM battery terminals directly under the driver's seat. If the charger is currently clamped onto the battery itself rather than the under-hood posts, that's worth changing — and if there's corrosion specifically at the under-hood posts (which route back to the actual battery), that alone could be contributing to what's being seen.

Don't rule out a genuinely bad battery

A charger cycling off repeatedly is also just standard protective behavior when it detects a bad or shorted cell — independent of any SAM/ground-strap issue. If the battery itself is the root cause, the electrical troubleshooting above would be a wasted detour, and the replacement plan is the more direct path. The steps below triage the battery in place to figure out which one you're actually dealing with.

Triage steps — battery in place

1. Visual inspection first (2 minutes, no tools beyond your eyes)

Case: any bulging, swelling, or distortion? AGM batteries that have been overcharged or overheated swell — a swollen case is a bad battery, full stop, don't bother testing further.

Terminals: corrosion (white/greenish crust), looseness, or visible damage at the post or clamp.

Leaking/weeping: AGM is sealed and shouldn't leak; any moisture around the case seams is a bad sign.

Date code: check the label for a manufacture date — helps you judge whether "just genuinely worn out" is plausible given the vehicle's age.

If the case is swollen or leaking, stop here — replace it, no further testing needed.

2. Resting voltage (multimeter, no charger or load connected for at least 1–2 hours first)

With the battery sitting untouched — charger and any vehicle load off it — read voltage across the terminals:

| Reading | Interpretation |
| --- | --- |
| ~12.6–12.8V | Fully charged, healthy |
| ~12.2–12.4V | Partially discharged — normal-ish, not a red flag alone |
| ~11.5–12.0V | Significantly discharged |
| ~10.5V or lower | Strong indicator of a shorted/dead cell — a 12V AGM has 6 cells at ~2.1V each, so losing one cell drops total voltage by roughly that much |

A reading in the ~10.5V range essentially confirms a bad cell on its own, independent of anything else below.

3. The decisive test — isolate the battery from the vehicle entirely

This is the single test that tells you which theory (bad battery vs. vehicle wiring/SAM fault) is correct, and it needs no special tools beyond what you already have.

1. Fully disconnect the battery from the vehicle — negative terminal first, then positive (standard safety order) — and move it a few inches clear of the vehicle's cables so there's no chance of accidental contact.

2. Connect your 6A charger directly to the isolated battery only, with nothing else attached.

3. Watch what the charger does:

If it still cycles on/off every ~30 seconds with the battery completely isolated → the fault is inside the battery (a bad/shorted cell). The vehicle wiring and SAM module are not involved. Skip the SAM/ground-strap diagnostics entirely and go straight to the replacement plan.

If it charges normally/steadily once isolated (no cycling) → the battery itself is fine, and the cycling + wiper activation only happens when connected to the vehicle. That confirms the SAM/ground-strap/wiring explanation above — proceed with the diagnostic sequence in that section instead, and hold off on replacing the battery.

(Note: wipers obviously can't activate while the battery is disconnected from the vehicle — that part of the symptom is expected to disappear during this test regardless. The charger's cycling behavior in isolation is the data point that matters here.)

4. If the isolation test points to a bad battery, confirm with a load test if you have the means

A battery can read an acceptable resting voltage but still collapse under real load if a cell has failing capacity rather than a hard short. If you have a load tester (or a charger with a test function), a healthy 12V battery should hold above ~9.6V for 15 seconds at its rated CCA per the standard SAE test. A battery that reads fine at rest but drops well below that under load confirms the cell is failing even without a dead-short symptom.

5. If the battery tests fine, clean before reconnecting

Before reconnecting a battery that passed isolation testing, clean the terminal posts and the vehicle-side cable clamps (wire brush + dielectric grease) — corrosion right at that connection point can itself cause exactly this symptom pattern and is worth ruling out before moving on to the SAM module deeper in the wiring.

Sources

SprinterRVDesk ("Mercedes Sprinter Wiring Problems: Ground Straps, SAM Faults, and the Gremlins That Fool Everyone"), Class B Forums (VS30 coach battery maintenance thread — charging connection point), MercedesMedic and BenzWorld forum threads (SAM module function and location), general battery-charger troubleshooting sources (cycling behavior with a bad/shorted cell). A directly on-point sprinter-source.com thread ("Wipers will not turn off") was found but not accessible to fetch — worth checking directly for another data point from other 907/VS30 owners.