# 27R Rear Suspension Upgrade Reference

Reference notes on a rear suspension upgrade (Hellwig sway bar + Agile-tuned Fox shocks) reported by another owner in a similar coach — captured here for evaluating whether it's worth pursuing on this 27R.

## Owner Report (source)

Posted to the View/Navion Motorhomes Facebook group (https://www.facebook.com/groups/ViewNavionMotorhomes/permalink/10163309569298359/):

> 2027-24R. With 4500 MB chassis and 4 cyl engine. After two trips, we decided to "upgrade" the rear suspension to improve ride and sway. Installed Hellwig 7777 sway bar and Agile 2.5 Dia Fox rear shocks. (Fyi: we had this done to our 2022-24D). After picking up the unit after the installation, both my wife and I noticed the improvement and are happy so far. Next trip is Sunday to make sure. An independent RV shop did the installation and only charged my 2 hrs of labor.

Key details:

Coach: 2027 Winnebago View, 24R floorplan — same generation/floorplan as our 27R

Chassis: Mercedes-Benz Sprinter 4500, 4-cylinder engine

Parts: Hellwig 7777 rear sway bar + Agile 2.5" diameter Fox rear shocks

Labor: ~2 hours, done by an independent RV shop (not a dealer)

Feedback so far: both owners noticed an improvement in ride quality and sway after the first drive; a further trip was planned to confirm

This same owner had already done the identical upgrade on a previous coach (2022 View 24D) — this is a repeat purchase from someone with direct before/after experience on two different Views

## Part Research

### Hellwig 7777 — Rear Sway Bar

| Field | Detail |
| --- | --- |
| Fitment | 2019–2023 Mercedes-Benz Sprinter 3500 and 4500 — confirmed fits this coach's chassis |
| Bar diameter | 1-1/2" |
| Install | Bolt-on |
| Warranty | Lifetime (manufacturer) |

Sources: Vivid Racing listing (https://www.vividracing.com/hellwig-rear-sway-bar-kit-mercedesbenz-sprinter-3500-4500-20192020-p-153666471.html), PartsHawk (https://partshawk.com/hellwig-7777-suspension-stabilizer-bar-assembly.html)

### Agile Off-Road — Fox 2.5" Rear Shocks

| Field | Detail |
| --- | --- |
| Shock body diameter | 2.5" |
| Valving | Two tunes available — Light (under 6,500 lbs rear axle weight) or Heavy (6,500+ lbs) |
| Design basis | Heavy-duty Fox 2.5 Commercial Series shock (originally designed for heavy, high-mileage semi-trucks), with a custom-length shaft and Agile-tuned digressive valving for the Sprinter rear suspension |
| Install | Bolt-on; requires factory 2" lift blocks (2WD models may need factory or Agile lift block kit) |
| Price (3500 RV/Class C kit) | $549.99/pair |
| Warranty | Fox Commercial Series Warranty (separate document from Agile) |

Source: Agile Off-Road — Fox 2.5 Rear Shocks, Sprinter 3500 RV/Class C (https://agileoffroad.com/products/agile-fox-2-5-rear-shocks-mercedes-sprinter-3500-rv-class-c)

## ⚠️ Open Question: 4500 Chassis Compatibility

The specific Agile "RV/Class C" Fox 2.5 kit found above is explicitly listed as fitting Sprinter 3500 only — the product page states it does not cover the 4500. This doesn't match the Facebook poster's own stated chassis (4500), so one of the following is likely:

Agile sells a separate, differently-listed SKU for the 4500 that didn't surface in this search, or

The 3500 and 4500 share enough rear-axle/leaf-spring architecture that the same shock physically fits both even though Agile's product page doesn't say so, or

The independent shop sourced a different/compatible shock rather than this exact listed kit.

Before ordering: confirm 4500 fitment directly with Agile Off-Road (or the shop that did this install, if we can identify them) rather than assuming the listed 3500 kit is a drop-in fit.

## Next Steps

1. Confirm the correct Agile Fox 2.5 SKU (or equivalent) for a Sprinter 4500 chassis.

2. Get a quote from a local independent RV/Sprinter suspension shop for the Hellwig 7777 + confirmed-correct Agile shocks, parts and labor.

3. Follow up on the Facebook thread for the poster's after-second-trip impression before committing.

4. Decide whether to pursue this for our own 27R.