# 2027 Winnebago View (27R) — 12V Fuse/Breaker Panel Location

Sources:

Winnebago_View_2027_Operator_Manual.pdf (Winnebago_View_2027_Operator_Manual.pdf) — Section 7 (Electrical)

Wiring_Diagrams/Wiring_Diagram_12Volt.pdf (Wiring_Diagrams/Wiring_Diagram_12Volt.pdf) (point-to-point 12V wiring, View/Navion 524R2)

Wiring_Diagrams/Body_12V_Wiring_Installation.pdf (Wiring_Diagrams/Body_12V_Wiring_Installation.pdf) (body 12V installation drawing)

Wiring_Diagrams/Wire_Identification_Chart.pdf (Wiring_Diagrams/Wire_Identification_Chart.pdf) (wire color/code legend)

## The Question

Where is the fuse panel for coach circuits (interior lights, etc.)? The larger fuses/breakers under the passenger seat were already found — is there a separate, smaller panel for lighting and similar circuits?

## Finding #1 — What's Under the Passenger Seat Is the Main Coach 12V Panel

The panel found under the passenger seat is the coach's primary 12V distribution/protection panel. Winnebago's documentation refers to it two ways:

Operator's manual: "Automotive Chassis 12-Volt Circuit Breakers" — "Located on outboard side of passenger seat" (Section 7, p. 7-17)

Wiring diagram: `PANEL ASM-CIRCUIT BREAKER`, labeled "Circuit Breaker Coach Panel" on the schematic (`Wiring_Diagram_12Volt.pdf`, sheet showing connector `T102`)

Wire Identification Chart: code "Z" — "Overcurrent Protection (45 or 55A breaker typ)" → "Coach 12V Panel"

What's inside this panel per the wiring diagram:

Coach Batt Fuse

Chassis Batt Fuse

FCC 2 Fuse (2 Amp)

Slideout breaker (LLK)

Wiring for the electric step relay, stabilizer chime/indicator light, and coach/chassis battery connections

This is the coach's main 12V distribution panel — not a secondary panel. There is no separate "second, smaller" fuse panel of this type documented elsewhere in the manual or diagrams.

## Finding #2 — Interior Lighting Runs Through a CZone Digital Switching System, Not a Traditional Fuse Box

This is the more important finding: this coach does not use a bank of small blade fuses for interior lighting and similar 12V circuits. Instead, those circuits are digitally switched and electronically protected by a CZone system, the same backend the WinConnect touchscreen controls.

The following modules are called out explicitly in `Body_12V_Wiring_Installation.pdf` (installation sheet 6 of 16):

| Module | Part Number |
| --- | --- |
| MODULE-CZONE RV1 (central digital switching module) | 000262772 / 353697-01-000 |
| MODULE-RV-C COACH MANAGEMENT | 000272790 / 354334-01-000 |
| MODULE-RV-C INTERFACE, TRUMA | 000269591 / 353636-01-000 |
| CONTROLLER-INTERFACE, SWITCH, CAN, CZONE SI (switch interface) | 000268667 / 353306-01-000 |
| CONTROLLER-LIGHTING, RGB | 000298374 / 360157-01-000 |
| CONTROL-CHARGE, SOLAR, 30 AMP MPPT | 000221265 / 342476-01-000 |

The point-to-point wiring diagram (`Wiring_Diagram_12Volt.pdf`, sheet with connector RV1, a 133-pin connector) shows the RV1 module's outputs staged electronically by current rating rather than by individual replaceable fuses:

35A outputs (H-Bridge)

5A outputs (H-Bridge)

0.5A outputs

"H-Bridge" outputs are electronically switched and current-limited/self-protecting — there is no physical fuse to inspect or replace per circuit. Lounge switches, accent lighting, and similar loads are wired directly into this module (e.g. `SWITCH-LOUNGE 1`, `LIGHT-MAIN ACCENT 1` connect into the RV1/coach harness in the diagram).

## Practical Implication

If an interior light or small 12V accessory stops working, there is likely no blown fuse to find or swap for that circuit. Instead:

1. Check the WinConnect touchscreen first — CZone-based systems report overcurrent conditions and circuit faults digitally (an alert/fault indicator on the relevant screen), rather than popping a visible fuse.

2. If a fault is shown, follow the WinConnect prompt or power-cycle the affected circuit from the touchscreen.

3. Only the items actually protected by the passenger-seat Coach 12V Panel (coach/chassis battery feeds, slideout, electric step, stabilizers) use conventional fuses/breakers you can physically inspect and replace.

## Open Gap

Neither the operator's manual nor the wiring diagrams give a specific physical mounting location (which cabinet/compartment) for the CZone RV1 module itself — these are schematic/wiring documents, not floor-plan/installation-location drawings. To physically locate the RV1 module:

Check the InfoCase for a CZone or Winnebago digital-switching-system supplement (not included in the base operator's manual).

The wiring diagram sheet ordering places the RV1/CZone harness sections adjacent to the Coach 12V Panel sections, suggesting they may be mounted in the same or a nearby compartment — but this is an inference from document layout, not a confirmed location.