# Winnebago 27R PDI Walkthrough Checklist

Purpose: a working checklist for delivery day so nothing gets missed. Pairs with Winnebago_27R_Operations_Guide.md (Winnebago_27R_Operations_Guide.md) (systems reference) and Winnebago_27R_New_Owner_Orientation.md (Winnebago_27R_New_Owner_Orientation.md) (questions to ask/have demoed). This doc stays action-only — look up the "why" there.

Ground rule (iRV2, Alliance RV Owners, Jayco Owners forums): don't sign final paperwork or release payment until everything below is satisfactory — you lose most leverage once you sign. Bring your own copy of this checklist rather than relying on the dealer's PDI form; theirs reportedly covers far fewer points (~100) than a thorough owner-driven walkthrough.

## Before delivery day

☐ Confirm appointment length — block 2–3+ hours minimum, a half-day if you can; don't let it get rushed

☐ Ask Poulsbo to de-winterize / have all systems live (water, power, propane) before you arrive

☐ Ask Poulsbo to power on the fridge/freezer the day before — it needs a full 24 hours to reach temperature

☐ Confirm who's doing the walkthrough — request the factory-trained tech, not sales

☐ Bring:

Phone + charging cable (Winnebago Connect pairing, USB port test)

Notebook/this checklist, camera

Small plug-in device (night light/charger) to spot-test 110V outlets

GFCI tester, if you have one

Instant-read thermometer (hot water and fridge/freezer temps)

Small induction-compatible pot or pan (to actually confirm the cooktop heats)

Bubble level (to independently verify the auto-leveling system)

## Paperwork to collect

☐ Final bill of sale matching the negotiated out-the-door number

☐ Title/registration paperwork

☐ Factory warranty documents — coach and Mercedes chassis (separate warranties; see Extended Warranty (#extended-warranty) below for the service contract paperwork)

☐ Owner's manuals: coach, Mercedes Sprinter chassis, appliances (Truma, Lithionics, generator, MBUX)

☐ Build sheet / window sticker for this VIN, itemizing factory options

☐ Open recalls: ask whether any chassis recalls apply to this VIN and are completed — e.g. RCRIT-20V468, a 2020–2021 Sprinter seat-wiring-harness recall

☐ Federal certification / weight sticker (driver-side doorjamb) — photograph it. This is the authoritative GVWR/GAWR/UVW for your unit, not a spec-sheet figure

☐ Keys: ignition, entry door, storage bays, gas cap — confirm count, get spares cut if only one set

☐ Winnebago Connect account setup / login credentials

## Exterior walkthrough

☐ Baseline photos/video first, before touching anything else — every exterior panel, the roof if accessible, every interior room/cabinet. This is your dated "before" record for any later dispute (damage claim, punch-list disagreement, insurance).

☐ Full walk-around for panel damage, scratches, or delamination

☐ Tires: DOT date codes, pressure, matching brand/size, spare present and correct pressure, tread checked for embedded nails/debris

☐ Exterior lights: running, brake, turn, backup

☐ Slide-out seals: intact, no gaps or tears

☐ Awning: full extend/retract, fabric condition, LED lighting

☐ Gear garage: both doors open/close/latch, L-track intact, light and 110V/12V ports work

☐ Exterior shower: hot and cold

☐ City water inlet, fresh fill, dump valves: cap correctly, dust caps present

☐ Propane tank: level and connections

☐ Entry door: alignment, latches without forcing — have the tech show the "push near the handle" technique (see New Owner Orientation → Entry door & general handling (Winnebago_27R_New_Owner_Orientation.md#entry-door--general-handling))

## Undercarriage & roof

The two areas easiest to skip on a normal walk-around.

☐ Roof: seams/sealant checked for bubbles or lifting if safely visible; press up on interior ceiling panels/vents for soft spots (water intrusion sign)

☐ Frame rails, crossmembers, body mounts: no rust-through, cracks, or loose brackets

☐ Leveling jack brackets and hydraulic lines: securely bolted, no leaks or weeping

☐ Tank mounting straps: level and secure — recheck once tanks are filled (see Kitchen and Water system below)

☐ Exhaust, chassis and generator: hangers intact, no rattling, scraping, or soot staining

☐ Suspension: leaf springs/shocks intact, hardware present

☐ Wiring harnesses and hoses: clipped/secured, not resting on anything hot or sharp

☐ Driveshaft and U-joints: no obvious play or excessive wear

☐ Brake lines/hoses: no damage, cracking, or leaks

☐ Propane line routing: secure, no kinks or damage

☐ DEF tank and lines (separate from the diesel tank): secure, no damage or leaks

## Chassis / dash walkthrough

☐ Odometer matches paperwork

☐ Engine fluids topped off: oil, coolant, power steering, brake, washer fluid

☐ Dash warning lights: none illuminated at startup

☐ DEF level and top-off point confirmed (see Operations Guide → Chassis & driving (Winnebago_27R_Operations_Guide.md#chassis--driving-mercedes-sprinter) for tank capacity/jug guidance)

☐ MBUX paired to your phone, WiFi hotspot tested

☐ Fuel level reasonable for the drive home

☐ Mirrors, digital rearview mirror, backup camera: test at multiple speeds, not just Reverse (see New Owner Orientation (Winnebago_27R_New_Owner_Orientation.md))

## Interior systems

Grouped by system below — operate everything live, don't just glance at it.

### Power & electrical

☐ Winnebago Connect touchscreen: cycle lighting, climate, water levels, battery status — for climate, actually switch on heat and confirm the propane furnace fires, not just that the touchscreen shows a heat icon

☐ All 110V/USB outlets live, GFCI outlets trip/reset test

☐ Breaker panel: labels match actual breakers (trip one to confirm)

#### Shore power

☐ Battery charges from shore power — confirm rate/status changes

☐ Air conditioner: cooling and heat pump (heating) modes both work

#### Generator

☐ Start it, run a few minutes, confirm it carries AC/high-draw loads; note the hour meter (should read near zero)

☐ Battery charges from the generator, separately from shore power

☐ Air conditioner: repeat cooling/heat pump test on generator power only, to confirm it can start and carry the load

#### House battery

☐ State of charge display reasonable

☐ Solar charging indicator shows input (if sunny)

☐ Inverter: run a 120V load off battery only, generator/shore off

### Kitchen and Water system (Andrea)

☐ Water heater: hot water in ECO and Comfort modes — verify the actual temperature difference with your thermometer

☐ Fill the fresh tank, run every faucet (including outside shower) — check under sinks, behind panels, and at fittings for leaks while flowing

☐ Water pump: cycles on, builds pressure, shuts off cleanly — no continuous running or chatter (purge air at every faucet first if the unit was just de-winterized)

☐ Toilet, black/gray tank level sensors correct; dump valves — no leaks while draining

☐ Fridge/freezer actually cold — verify with your thermometer (~35–38°F fridge, ~0–5°F freezer)

☐ Induction cooktop and LP burner both ignite — confirm heat with your own induction-compatible pot

☐ Microwave/convection oven: powers on, runs a test cycle

☐ Kitchen USB-A/USB-C outlets charge a real device, not just show power

☐ Every drawer/cabinet: open fully, check for shipping debris or shifted/damaged contents

☐ Windows and blinds/shades: every window opens/closes, screens intact, shades cycle

### Leveling, slide-out & bed

☐ Leveling jacks: full auto-level cycle, verify level with your own bubble level (not just the touchscreen chime), then test manual override each corner

☐ Slide-out: full electric cycle, smooth operation, trim doesn't bind; have the tech show the manual/hand-crank override

☐ Flex Bed: convert twin → combined and back, with the tech demonstrating the mechanism hands-on

### Safety & accessories

☐ Smoke, CO, and LP gas detectors: test buttons, confirm not expired

☐ Included accessories present: sewer hose + support/ramp, fresh water hose, shore power cord, wheel chocks, leveling blocks, entry step, microwave accessories (turntable, screens, manual), tool kit or spare fuses

## Confirm against build sheet

☐ Generator: confirmed 3,200W propane, not diesel — brand still unconfirmed, worth a glance at the nameplate (see Operations Guide → Power system (Winnebago_27R_Operations_Guide.md#power-system-house-batteries-solar-inverter-generator))

☐ Leveling jack brand: factory Equalizer/EQ Systems vs. any HWH upgrade

☐ GVWR/GAWR/UVW off the certification sticker, not a marketing page — Winnebago's own published figures for this floorplan have been inconsistent across sources, so the sticker is authoritative

☐ Rearview camera: reverse-only or live while driving, and at what speed cutoff (forums report ~15 mph)

## Wrap-up

☐ Photo/video the tech's demonstrations (leveling cycle, slide operation, Flex Bed conversion) if allowed — separate from the baseline condition photos taken at the start

☐ Ask what maintenance is due in year one — chassis service intervals, filters, seals/seams recheck, anything time- or mileage-based

☐ Plan to get the loaded coach weighed at a CAT scale, per-axle, before any towing — real tow margin is GCWR (15,250 lb) minus actual loaded weight, not a curb-weight estimate

☐ Confirm the dealer's punch-list process in writing, with a timeline

## Extended Warranty

☐ Extended service contract documents in hand, if purchased

☐ Contract administrator name in writing — the entity that actually pays claims, not just the marketing brand (e.g. "Winnebago XtraRide" is a brand; the administrator behind it, likely Protective Asset Protection, is what matters)

☐ Term purchased matches what was negotiated — see Negotiation Playbook → Extended warranty negotiation (Winnebago_27R_Negotiation_Playbook.md#extended-warranty-negotiation) for the pricing benchmarks

☐ Coverage split confirmed: coach warranty vs. Mercedes chassis warranty vs. this contract, and who to call for each

☐ Exclusions confirmed — diesel/Sprinter contracts vary on whether chassis electronics, turbo, and emissions systems are covered