House2Start Trickle Charger — Install Plan (2020 Winnebago View)

Purpose: keep the chassis/starter battery topped off from the house battery bank while parked/stored, so what just happened (chassis battery found dead at 3.5V, see Winnebago_View_2020_Chassis_Battery_Replacement.md) doesn't recur from unmonitored drain.

What it is

House2Start — a modern replacement for the discontinued Trik-L-Start. A small, permanently-installed 12V DC-DC trickle charger that runs a true 4-stage charge profile (not a dumb float) to maintain the chassis/starter battery off the house bank. Compatible with Lead-Acid, AGM, or Lithium chemistries — relevant since you're going from an H6 to a proper H8 AGM on the chassis side.

Price: $99

Amperage: 2A charger — enough for maintenance/trickle duty, not a bulk charger

Size: small form factor, fits tight spaces

Wiring: pre-attached 18" yellow/blue/black wires simplify connection to the target device

Where it installs on this vehicle: the Mastervolt Chargemate

House2Start explicitly calls this out as the easiest installation point for this exact vehicle type — "installed in most 2020+ Winnebago Navion/View/Era (Mercedes chassis) motorhomes" — which is this 2020 View.

Location: Mastervolt Chargemate, under the passenger seat

The Chargemate is a current-limiting battery combiner — House2Start connects directly onto its power lugs that lead to the house and starter batteries, rather than needing a separate wiring run to each battery.

Generation-specific requirement — check before wiring

Gen2b: requires installing the included diode on the blue wire when connecting to a Mastervolt Chargemate.

Gen2c or later (anything shipped after Dec 22, 2025): diode requirement was eliminated for Chargemate installs.

Check the sticker on the unit itself to confirm which generation you actually received before assuming either way — don't guess from order date alone.

Install steps

1. Locate the Mastervolt Chargemate under the passenger seat.

2. Identify the power lugs on the Chargemate connected to the house battery bank and to the starter/chassis battery.

3. Verify with a multimeter before connecting anything — House2Start's own installation guide recommends measuring voltage at the actual battery terminals first, then cross-checking against the Chargemate's connection points, rather than trusting lug labeling alone.

4. Check the House2Start unit's generation sticker (2b vs. 2c+) to determine whether the diode step applies.

5. If Gen2b: install the included diode on the blue wire per the unit's documentation before making the final connection.

6. Connect the pre-attached yellow/blue/black wires to the corresponding Chargemate lugs (house+, starter+, ground) per the House2Start manual.

7. Mount the unit securely in the available space near the Chargemate — small form factor, but still secure it so it can't vibrate loose or contact anything it shouldn't.

After install

Confirm the chassis battery's state of charge trends upward/holds steady over a period of parked/unplugged time (a few days) rather than drifting back down — this is the actual proof the unit is doing its job.

Since this runs off the house bank, keep an eye on house battery state of charge too if boondocking for extended periods — a 2A draw is small but not zero.

Sources / open questions

Compiled from house2start.com (product page, installation guide, FAQ) and a Sportsmobile/Winnebago Revel Forum reference confirming the Mastervolt Chargemate as standard equipment on 2020+ View/Navion/Era. Not confirmed: the underlying electrical reason the diode is needed on Gen2b units specifically — House2Start's public documentation states the requirement without explaining the rationale. Worth asking House2Start support directly if you want that detail before wiring it in, rather than guessing.