# Isolating a Continuous 60W Parasitic Draw

Generated from `27R_60W_Draw_Isolation_Checklist.yaml` via `troubleshooting_doc.py` — edit that file, not this one.

Start with every switch believed OFF. This checklist confirms that's actually true, then reads the true baseline and isolates further if the draw remains.

What you need: WinConnect touchscreen (Energy Management screen), passenger-side compartment access, water heater exterior access panel, entry/galley walk-through.

*Several items below have a physical switch and a separate software toggle that can disagree with each other — that mismatch is the most common cause of an unexplained draw.*

## Part 1 — Confirm Everything Is Actually Off

Work through in order — each item lists both the physical check and the software confirmation.

### ☐ 1. Shore power and generator disconnected

Shoreline power cord fully unplugged and coiled back into its compartment (driver's side).

Generator confirmed OFF (WinConnect generator screen, plus listen/check for exhaust).

Why this matters: If shore power or the generator is live, you're not reading a true battery drain.

### ☐ 2. Inverter/charger disconnect switch — TOP SUSPECT

Physical switch (passenger-side compartment, near the inverter/charger unit) is OFF.

WinConnect → Energy Management screen also shows the inverter OFF (not just “on but inactive”).

Why this matters: This switch is separate from the software toggle. Left on while the app shows “off,” the inverter's standby electronics can keep drawing power — plausibly the full 60W by itself.

### ☐ 3. Water heater

Physical switch behind the exterior access panel is OFF (green indicator light off).

WinConnect → Water Systems screen confirms OFF — not just Economy mode (which still cycles the recirculation pump).

### ☐ 4. Tank heating pads

WinConnect → Water Systems screen — pad toggle(s) confirmed off.

Why this matters: Thermostatically controlled — can silently re-engage in cold weather.

### ☐ 5. Refrigerator

Control panel/thermostat fully OFF (not just turned down to “1” — that's still running).

WinConnect Home screen (if it shows fridge status) agrees.

Why this matters: This is a 12V DC compressor refrigerator — draws directly off the house battery.

### ☐ 6. Winegard router / WiFi booster

Dedicated power switch (separate from interior light switches) confirmed off.

Why this matters: Exact location isn't specified in the manual — check near the Winegard antenna control panel.

### ☐ 7. Interior lights and 12V accessories

Walked the length of the coach — no interior/reading/accent lights lit.

Any actively-charging USB devices unplugged from 12V USB outlets.

### Expected residual load — don't chase these

Propane gas leak detector & CO detector — always powered by design; should be well under 1–2W combined.

CZone/RV-C control modules (main switching module, coach management, Truma interface, lighting controller, solar charge controller) — stay powered whenever the master battery switch is on. Normal combined standby should be low single-digit watts.

## Part 2 — Read the True Baseline

With every Part 1 item confirmed off (master House/Coach Battery Disconnect switch still ON — you need this to read anything):

1. Open WinConnect → Energy Management screen.

2. Read the Usage value — this is your true “everything off” baseline.

| Reading | What it means |
| --- | --- |
| Near 0W | Found it — whichever item in Part 1 was actually still live (most likely the inverter disconnect switch mismatch) was the cause. |
| Still ~60W | The draw is coming from something not covered by a normal user-facing switch — continue to Part 3. |

## Part 3 — If the Draw Persists: Further Isolation

1. Re-confirm each Part 1 item once more — double-check the inverter disconnect switch especially.

2. Inspect the Coach 12V Panel (passenger-side compartment) for any breaker/fuse that looks tripped, warm, or abnormal. Do not pull the Coach Batt Fuse or Chassis Batt Fuse to “test” — that kills power to the monitor you're trying to read.

3. If nothing explains it: likely a stuck relay in a CZone/RV-C module, or a wiring short/chafe point.

4. Use the Wire Identification Chart and wiring diagrams to trace circuits with a multimeter/clamp meter, or contact a dealer/mobile RV electrician — a draw surviving every user-accessible off switch usually means a component-level fault.

Companion documents: 27R_12V_Fuse_and_Breaker_Panel_Location.md/.docx (Coach 12V Panel contents and the CZone digital switching system) | 27R_2000W_Inverter_Operation.md/.docx (inverter operation and disconnect switch location)